DRAFT VERSION 2.0

of the

GOES Data Collection Platform Radio Set (DCPRS) CERTIFICATION STANDARDS

at 300 bps and 1200 bps

NOAA / NESDIS

Draft release date: May 26, 2006

TABLE OF CONTENTS

***THE TABLE OF CONTENTS WILL BE REVISED WHEN THE CHANGES TO THE CONTENT HAVE BEEN FINALIZED***

GOES DCS 300/1200 BPS DCPRS CERTIFICATION STANDARDS

FOR SELF-TIMED, RANDOM REPORTING, and INTERROGATE OPERATION

Introduction

DCPRS Certification is achieved by demonstrating that a radio transmitter fulfills each of the requirements set forth in this document.  NESDIS certification is "type certification", wherein a representative production unit is tested and found to fulfill all stated requirements.  NESDIS certification of individual production units (those having the same model number) is not required.  As a standard, this document represents mandatory requirements—waivers will not be accepted.

To obtain “Type Acceptance” the manufacturer shall submit the DCPRS schematics, data flow chart, electronics parts design data, unit and system test data, and perform tests to demonstrate that each requirement herein is met.  This includes but is not limited to analysis of the DCPRS’ design and performance characteristics, performing unit tests at room temperature, and over a range of temperatures and power supply (battery voltage) variations.

Certification testing shall be run with standard laboratory test equipment and with a NOAA supplied Test Receiver/Demodulator.  This test unit includes a computer output port from the demodulator for test data extraction points from the unit under test.  The output port is accessed by the use of a “dumb” terminal which is obtained by running a terminal emulation programs such as BitCom, COMMIT, etc. through an IBM desktop or laptop PC.  The test demodulator has other access points for “I” and “Q” phase detector measurements.  Test guidelines, procedures, etc. are described in an appendix of this document.  The NOAA Test Receiver/Demodulator will be loaned for a period of up to sixty (60) days to manufacturers after the required documentation and design data requirements in Section 1 have been accepted and/or approved by the NOAA NESDIS Certification Official.

Manufacturers are required to supply all other test equipment needed to demonstrate compliance with the certification requirements.  All test equipment to be used must be identified in the manufacturer’s initial request for DCP certification and have verified calibrations to an established test laboratory.  A list of the recommended test equipment, a typical test set up, and some of the capabilities of the NOAA provided test set are included in Appendix A.  Specific test channels for use by manufacturers shall be assigned by NESDIS OSDPD as required.

Document Organization

The GOES DCPRS certification requirements are set forth in the following four sections.  The first section identifies the DCPRS certification documentation required.  The second section defines the DCPRS certification reporting mode requirements.  The third section focuses upon the DCPRS transmit data format requirements which are not considered to be temperature dependent.  The fourth section involves requirements for which performance may vary over temperature and power supply variation such as output power, frequency stability, modulation stability, carrier phase noise, transmit spectrum, etc.

SECTION 1 - DCPRS Certification Documentation Requirements

1.1	Required Documentation

Manufacturers shall submit the following documentation at least 60 days prior to beginning the formal DCPRS certification testing.  While NESDIS may review and comment on this documentation, all documentation is considered ‘PROPRIETARY’ for government eyes only and not available to any other party, unless so identified by the manufacturer.

- DCPRS Model Number with its respective data and/or specification sheet(s)

- DCPRS electrical and electronic circuit schematics.

- DCPRS software flow-charts that identify how the DCPRS reporting method(s) - random, self-timed, interrogated,  message formatting/generation, frequency and time stability functions, Fail-safe operation, other functions are fulfilled.

- DCPRS antenna gain, polarization, axial ratio, and VSWR data.  This data shall be used with the power amplifier output to determine the DCPRS EIRP.

- Manufacturers Proposed Test Procedures including test data sheets.

- DCPRS oscillator aging analysis data to demonstrate that the specified aging requirements are met.

- Preliminary DCPRS Transmit Spectrum (see paragraph 4.5)

1.2	Emission Designators

The official emission designations for the DCPRS transmissions are provided below and shall be used whenever an emission designator is required.

- For 300 bps user data rate:  300HG1DEN.

- For 1200 bps user data rate:  1K20G1DEN.

- If any DCPRS is able to operate at 100 bps, in accordance with previous certification standards, the applicable emission designator is 400HG1DBN.

1.3	Nameplate Information

Each DCPRS shall include a durable nameplate on its outer surface that shall contain the following information as a minimum:

- Manufacturer’s name, model number, and serial number

- Certification date for this model

- Defined input voltage range (see paragraph 4.0)

- Approved antennas for use with this DCPRS

- Emission designator(s) applicable to this model

SECTION 2 - DCPRS Data Rate and Operating Mode Requirements

For High Data Rate (HDR) operation a DCPRS may be designated for either 300 bps or for 1200 bps.  Further the DCPRS manufacturer shall clearly state the reporting mode(s) for the Model/unit (i.e.  Self-timed, Random, or Interrogate).  A combination of two operating modes is permitted providing this is so identified and that all applicable mode requirements are met.  The DCPRS certification official will identify specific channels, GOES ID/DCP Address, and time slots as needed for any ‘on-the-air’ or ‘GOES testing.’

2.1	DCPRS Self-Timed reporting Mode Accuracy

All DCPRS transmissions shall control the start time of their initial carrier-only period to be within less than or equal to ±0.25 seconds from the assigned reporting time (referenced to UTC) over the full range of operating conditions.

2.1.1	Inhibiting Transmissions

The DCPRS shall include a method to calculate the amount its internal clock differs from UTC over the full range of operating conditions.  Should the DCPRS clock differ from UTC by more than ±0.25 seconds, the transmissions shall be inhibited.

2.1.2	Proof of Performance

During Certification tests the manufacturer shall show how the difference between the internal clock and UTC is calculated and shall show how the requirement of 2.1.1 will be met.  The manufacturer shall demonstrate the basic factor(s) such as drift rate, used in the calculations.

2.2	DCPRS Random Reporting Mode Requirements

For random reporting certification manufacturers shall demonstrate that the DCPRS transmits at a maximum message length of 3 seconds for 300 bps and 1.5 seconds for 1200 bps.  The same algorithm and/or processes used for 100 bps certification may be used.  Definitions of pseudo binary and other concepts are included in Appendix B.  Random reporting certification testing requires that manufacturer’s demonstrate random message generation on an approved NESDIS channel and transmit DCP messages in the random mode for eight (8) or more hours with an average repeat interval of every 15 minutes.  One of the data values to be transmitted in this testing shall be a message number counter.

2.3	DCPRS Interrogate Reporting Mode Requirements

For DCPRS  interrogate mode certification the unit shall be tested as a transmit/receive system. Thus not only all the DCPRS transmit but also the GOES Command Receiver requirements must be demonstrated.  The DCPRS command receiver requirements are set forth in a separate document (TBD).

For ‘on the air’ testing a reply message must be generated when the interrogate command is received on the appropriate interrogate DCP GOES ID and a data transmission must be sent on the proper channel.

SECTION 3 - DCPRS Data Format Requirements

3.1 	DCPRS Message Format

The format of all messages shall meet the following requirements.

Carrier

0.5s/

0.25s

Clock

States

3 ‘0-1’

FSS

15 bits

GOES ID

32 bits

Flag

Word

8 bits

DCP DATA

Max: 32,000 bits @ 300 bps

128,000 bits @ 1200 bps

EOT

Encoder

Flush

32 bits

Start of Scrambling and Coding

NOTES:	ASCII EOT use for ASCII and Pseudo Binary Formats

International EOT used for Binary Format

Actual Message Length may not Exceed Fail-Safe Limits

Carrier:	0.50 seconds for 300 BPS

0.25 seconds for 1200 BPS

Clock:   	3 symbol periods, first 0 degrees, then 180 degrees, then 0 degrees.

Frame Synchronization Sequence (FSS):

The following 15 bit pattern shall be sent at the appropriate symbol rate with "0" representing 0 degrees and "1" representing 180 degrees:

(MSB) 000011001001010 (LSB) The left most bit is transmitted first.

After the FSS is transmitted the GOES ID and all data shall be scrambled, and trellis encoded, as defined below.

GOES ID/DCP address:

31 bits plus an extra "0" inserted as the LSB to form 4 - 8 bits Bytes

The GOES ID is a 31-bit Bose-Chaudhuri-Hocquenghem (BCH) encoded address with a zero included as the 32nd LSB.  This address shall be transmitted as the first 4 bytes of the data in the message in exactly the same manner as all the other data bytes in the message.  For example, given the hex ID of CE 12 00 B8, the first byte transmitted is CE hex, followed by 12  Hex, followed by 00 hex, followed by B8 hex or:

11001110 00010010 00000000 10111000

Flag Word:  (LSB) 	Bit 1	spare, undefined

Bit 2	Clock updated since last transmission = 1, not = 0

Bit 3	Data Compression on =1, off = 0 Possible Future Enhancement

Bit 4	New Coding  on = 1, off = 0  Possible Future Enhancement (Code format not yet defined)

Bit 5	spare, undefined

Bit 6	= 1 if ASCII or Pseudo Binary, otherwise = 0

Bit 7	= 1 if Pseudo Binary or Binary, otherwise = 0

(MSB) 	Bit 8	Odd parity for ASCII formatted data

Any DCPRS certified to this standard that will also be able to operate at 100 bps shall employ the short preamble defined in the 100 bps Certification Standard.

3.2 	Data Scrambling

Starting with the first bit of the GOES ID, all DCPRS data shall be scrambled.  The DCPRS serial data stream shall be “exclusive ORed” (XOR) with the serial binary string represented by the table below.  It is shown as 40 bytes using hex symbols for convenience, and shall be used in a circular fashion throughout the message.

| 53 | 12 | 72 | B2 | 54 | 62 | AA | E4 | DB | A7 | 56 | 08 | A8 | 09 | B4 | BF | 61 | DC | 50 | E3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AB | 7F | 00 | 87 | 6D | F5 | 58 | CC | CF | 3E | E7 | 2A | 7E | 9B | 5C | 4D | CE | A5 | 3C | 0A |

Scrambling shall be performed on a byte-by-byte basis.  For example, the first byte of the GOES ID example, CE (hex), would be XOR with the first byte in the table, 53 (hex), i.e., the serial string 1100 1110 XOR with the serial string 0101 0011 to produce 1001 1101, or 9D (hex).  Then the second byte of the GOES ID, 12 (hex), would be XOR with the second byte in the table, 12 (hex), to produce 00 (hex), and so on.  After the last byte of the table 0A (hex), has been XOR with the 40th byte of the DCPRS message, the table would be cycled and the 41st message byte XOR with the first byte in the table, 53 (hex).  This sequence would continue as many times as required for the duration of the message, including the EOT and flush bits.

3.3 	Trellis Encoding

Figure 3 provides a Functional Block Diagram of the Trellis encoder required.

Figure 3.3  Trellis Encoder Functional Diagram

This logic generates sets of tri-bits, which are used to generate each 8- PSK symbol to be modulated.

Each data byte contains 8 bits.  These bits are split into four 2 bit pairs as shown in the table below.  Each 2-bit pair is fed into the trellis encoder in order.  Pair 1 is fed into the encoder, then pair 2, pair 3, and pair 4.  Then pair 1 of the next byte, pair 2, etc.

| MSB |  |  | LSB |
| --- | --- | --- | --- |
| Bits 7 & 6 | Bits 5 & 4 | Bits 3 & 2 | Bits 1 & 0 |
| Bit Pair 4 | Bit Pair 3 | Bit Pair 2 | Bit Pair 1 |

The trellis encoder takes a 2-bit pair and encodes it to a 3-bit symbol.  This 3-bit symbol is then mapped to one of the eight phases for transmission as shown in Table 3.6.  The initial state of the encoder shall be all zeros.

3.4 	Encoder Flush

At the end of the message after transmitting the EOT, an additional 32 zero (0) data bits shall be provided to flush the encoder and decoder.  After the 32nd flush bit is transmitted, the phase should be brought to zero and the carrier power shall be turned off.  The turn-off time shall be one millisecond, maximum.

3.5 	DCPRS Modulation Encoding

The transmitted data shall be phase mapped from the trellis encoder as follows:

| E-2 MSB Bit | E-1 Bit | E-0 LSB Bit | Phase Symbol Degrees |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 45 |
| 0 | 1 | 0 | 135 |
| 0 | 1 | 1 | 90 |
| 1 | 0 | 0 | 180 |
| 1 | 0 | 1 | 225 |
| 1 | 1 | 0 | 315 |
| 1 | 1 | 1 | 270 |

Table 3.5 Phase Encoding

3.6 	DCPRS Data Formats

Three data formats shall be demonstrated ASCII, Pseudo Binary, and Binary.  HDR pseudo binary shall be the same as for 100 bps DCPRS certification (see Appendix B).

3.6.1	Prohibited Characters

The requirement in previous versions of this document that transmission of certain specified control characters was prohibited, has been deleted.  Requirements for Binary Mode will be set when a binary format is proposed and accepted.  Manufacturers should be aware that the receivers are programmed to end reception of any message when an EOT character (defined below) is detected and may therefore wish to ensure an EOT is never transmitted in the body of a message.

3.6.2 	End Of Transmission (EOT)

ASCII and Pseudo Binary Format Mode - An EOT character, bit pattern 00000100, transmitted with the LSB first, shall be sent immediately after the last symbol of sensor data.  This bit pattern is an ASCII EOT with odd parity.

Binary Mode - An (EOT) code word, 0110 0011 1100 1010 1101 1101 0000 0100 with the LSB transmitted first, shall be sent immediately after the last symbol of sensor data.  This bit pattern is identical to that required for international DCPRS certification except an additional zero (0) is inserted as the MSB to provide an even number of bits to be sent to the encoder.

SECTION 4 - DCPRS Performance Requirements

DCPRS performance requirements shall be demonstrated over a -40°C to 50C temperature range and over a power supply voltage range that is defined by the manufacturer.  The DC power supply voltage shall be monitored both before and during a transmission and the RF transmission shall be inhibited if the defined range is exceeded.

4.1 	DCPRS Effective Isotropic Radiated Power (EIRP)

4.1.1 	RF Power Output

For 300 bps, the DCPRS shall operate with an EIRP of 48 dBmi ±1 dB.  For 1200 bps, the DCPRS shall operate with an EIRP of 51 dBmi ±1dB.  These limits shall be met under any combination of operational conditions.

NOTE:    The satellite transponder has been designed to operate over the uplink EIRPs listed above.  Levels higher that the maximum will tend to overload the transponder and are prohibited.  Levels less than the minimum may not provide an acceptable error rate when a large number of channels are simultaneously active.  The minimum is cited because it is the lower limit of the system design, but users are permitted to use lower EIRP at their own risk.

4.1.2 	DCPRS Antenna

4.1.2.1 Antenna Polarization

DCPRS antenna polarization shall be right-hand circular, according to IEEE Standard 65.34.159 and have an axial ratio not more than 6 dB on axis.

4.1.2.2 DCPRS Antenna Gain

The DCPRS antenna transmit gain shall be such that in combination with the DCP output power the maximum EIRP is not exceeded.

4.2 	GOES DCS Operating Frequency Requirements

4.2.1 	Operating Channels and Frequencies

The DCPRS shall be able to operate at any of the designated channel center frequencies.

For DCPRS operating at 300 bps:

The DCPRS shall be able to tune to 532 channels at 750 Hz increments, from 401.701000 MHz to 402.099250 MHz.  The channel 1 center frequency shall be 401.701000 MHz, channel 2 center frequency shall be 401.701750 MHz, channel 3 center frequency shall be 401.702500 MHz, etc., up to channel 532 at 402.099250 MHz.

For DCPRS operating at 1200 bps:

The DCPRS shall be able to tune the following sub-groups of channels in 2250 Hz increments.  The first such channel shall be channel number 2, then channel number 5, channel number 8, etc., up to channel number 530, at 402.097750 MHz.

The assigned DCPRS operating channel center frequencies for 300 bps and for 1200 bps operations are set forth in Appendix D.  All frequencies listed in Appendix D shall be provided for 300 bps operation.  Only those shown in bold shall be available for 1200 bps operation.  However, actual operation at 1200 bps will be authorized only on a subset of these potential channels by the Office of the NOAA GOES DCS Program Manager.

For certification testing, manufacturers shall demonstrate the synthesis of each of these over the entire range of the possible operation and at a minimum of five frequencies for 300 bps and five for 1200 bps selected at random by the NESDIS Certification Official.

4.2.2 	Frequency Stability, Long Term

The DCPRS output frequency shall be maintained to within ±125 Hz of the channel center frequency due to any combination of operational conditions and for any channel.

4.2.3 	Short Term Frequency Stability

The DCPRS output frequency shall maintain a short term stability of ±1 Hz/s under any combination of power supply variation or temperature as set forth in paragraph 4.0.  For certification testing the frequency shall be measured over 10 ten-second sample windows and the maximum difference over 100 seconds shall be recorded.

4.3 	DCPRS Modulation Output Symbol Rate

For 300 bps certification the output symbol rate shall be 150 symbols per second ±0.025%.

For 1200 bps certification the output symbol rate shall be 600 symbols per second ±0.025%.

For certification testing the DCPRS is to be set to transmit 0.5 second of carrier and then a continuous stream of “0-1” or 0 and 180 clock transitions at the respective symbol rate.  The symbol period is to be measured at the “I” output of the test demodulator.

4.4	DCPRS Phase Modulation and Noise

For certification testing phase modulation and noise measurements shall be referenced to the zero degrees modulation phase transmitted during the message preamble and perfect 45 degree increments around the 360 degree circle.

4.4.1	Carrier Phase Noise

The phase noise of the unmodulated carrier shall be equal to, or less than 2.0 degrees RMS, when integrated over the range 10 Hz to 150 Hz for 300 bps, and 36 Hz to 600 Hz for 1200 bps.  This limit shall be met under any combination of operational conditions.

4.4.2	Phase Modulation Bias

The worst case offset or bias of any of the 8-ary modulation points shall be equal to, or less than 1.0 degrees, under any combination of operational conditions.

4.4.3	Total Phase Error

The RMS phase error due to all causes except phase modulation bias, and under any combination of operational conditions, shall be equal to, or less than 2.5 degrees.  This shall include the contribution due to carrier phase noise per 4.4.1.

4.5	DCPRS Transmit Spectrum

The DCPRS shall be designed to be received by the CDA stations, which will include a RRC demodulator filter with an excess bandwidth factor, α, of 1.0, implemented as a fourth order FIR digital filter with 65 taps, or more.  (TBR)

When modulated with a random data stream and measured on a spectrum analyzer using a 1 Hz resolution bandwidth, the peak response of the undesired part of the emission, relative to the peak response in the necessary bandwidth (NB), shall be equal to, or better than the following limits.  (The NB is 300 Hz for 300 bps and 1200 Hz for 1200 bps.)

- At any frequency removed from the channel center frequency by more than 75% of the NB, up to and including 150%, at least 25 dB attenuation.

- At any frequency removed from the channel center frequency by more than 150%, up to and including 300%, at least 35 dB attenuation.

- At any frequency removed from the channel center frequency by more than 300% of the NB, at least 43 + 10 log (P) dB attenuation, where P is the total mean power in watts in the necessary bandwidth.  (NOTE: This includes harmonics and any spurious radiation.)

For certification testing, manufacturers shall use spectrum analyzer with the RBW and VBW set to 1 Hz and averaging on, to provide a smooth trace so the performance at each frequency span can be clearly seen.  The number of sweeps to be averaged may be limited to the number required to show visible improvement.

Spectral masks are provided for reference in figures 4.5-1 and 4.5-2.

MAXIMUM ATTENUATION REQUIRED VARIES WITH OUTPUT POWER:

-53 dB APPLIES FOR 10 WATTS

900

450

225

-225

-450

-900

Figure 4.5-1  Emission Mask for 300 bps Operation

MAXIMUM ATTENUATION REQUIRED VARIES WITH OUTPUT POWER:

-53 dB APPLIES FOR 10 WATTS

3600

1800

900

-900

-1800

-3600

Figure 4.5-2  Emission Mask for 1200 bps Operation

4.6 	Fail-safe Operating Requirements

An independent or separate fail-safe circuit shall be provided to prevent a DCPRS from operating in an uncontrolled fashion.  This independent circuit must “permanently” shut off the transmitter if either of the following two conditions is violated.

- Message is too long.

For 1200 bps transmissions the maximum message length shall be 128,000 bits

For 300 bps transmissions the maximum message length shall be 32,000 bits

If any DCPRS includes operation at 100 bps, the maximum message length at that user data rate shall be 9600 bits

These maximum message lengths are near 110 seconds, including the message format requirements.  If a message contains more than these numbers of bits, the fail-safe may be tripped.  If the total message length exceeds 110 seconds, the fail-safe shall be tripped.

- Message is sent too soon.  There shall be a minimum of 30 seconds off-time between successive transmissions.  If a second message is transmitted before 30 seconds has expired, then the fail-safe shall be tripped.

The fail-safe capability must be demonstrated over the full range of operating conditions.  Removal of DC power from the DCPRS shall not affect the operation of this function.

Notes:

1.	The above term “permanently” requires a manual intervention or reset of the DCPRS in order to restore the unit for operational use in the DCS.

2. 	The Inhibiting Transmission requirements of subsections 2.1.1 and 4.0 are specifically NOT included in this subsection.

APPENDIX A - Recommended Test Equipment and Test Set Up

The following test equipment or approved equal are recommended:

Spectrum Analyzer (SA) - The SA is needed to perform spectrum tests.  The SA must be able to measure to the third harmonic at 1206 MHz and have the Resolution Bandwidth, averaging capability, etc. necessary to demonstrate compliance with the required spectrum.

Frequency Meter (FM) - The FM is needed to measure transmit frequencies (401.7 to 402.1 MHz) and to measure the transmit symbol rate.  The FM shall be accurate to 0.001 PPM (parts per million).

Digital Multi-Meter (DMM) - The DMM may be used for power supply voltage, RMS response to measure phase noise, and other measurements as deemed appropriate.

RF Power Meter (RFPM) - The RFPM is used to measure the RF power amplifier output power.  The response needs to be to RMS power.  A Bird Wattmeter Model 43 with ±3 per cent accuracy at full range or approved equal is acceptable for these measurements.  The element power rating shall be such that the power measurements shall be in the top half of the range.

Signal Generator (SG) - The SG is to be used for mixing the 402 MHz signal to the 5 MHz IF of the test demodulator.  The SG phase noise shall be < 0.1 degree RMS.  An HP 8660C or approved equal is acceptable for this testing.

Environmental Test Chamber (ETC) - The ETC is used to control the ambient test temperature of the DCP unit under test.  A –40C to 50C range or greater is required.

General Purpose Oscilloscope - To measure or observe the relationship of “I” and “Q” signals.

Laptop or IBM PC - To interface with the NOAA provided Demodulator Test Set.

Low Frequency Signal Controller/Modulator - An HP 33120A or equivalent.  Used to generate modulation patterns and IF signals.

DCPRS Test Capabilities Required

During certification testing manufacturers must be able to disable or enable the DCP fail-safe circuitry as needed.  Further a number of test sequences must be available on the unit to test various DCP functions.  These include the following:

a) Carrier only 				carrier phase noise, frequency

b) Clock pattern “0-1"				spectrum and clock pattern check, symbol rate

c) Repeating short message sequence		format checks

d) Longer message, repeating pattern		modulation and Inter-symbol Interference check, power  measurement, spectrum

The typical test set up is shown in Figure A1.

Environmental Chamber

DCP

under

test

Power

Supply

DCP

Control

PC

O’scope

Frequency/

Period

Meter

Test

Demodulator

300 or 1200

bits/sec

I

Q

Wattmeter

With

Signal Tap

Dumb

Terminal

Signal

Generator

Spectrum

Analyzer

Phase Noise

Test Set

CONNECT

AS NEEDED

Vector Signal Analyzer

Test Set

The typical test set up is shown in Figure A1.

Some of the capabilities/features of the Test Demodulator are highlighted below:

Unique Software Functions For the Test Set Demodulator

The demodulator makes signal quality measurements of all the signal variables of power, frequency, and phase.  In the test case the measurements are required to be made in different sequences and accuracy.  In the test case the noise floor is the base line phase noise of the system since the S/N typical of live signals is not applicable.

Power Measurement

The total signal power out of the DCPRS shall be measured without external filtering.  Power measurements shall be made specifically for unmodulated carrier, clock, and random 8 PSK modulation under standard conditions.  Accuracy shall be ±3% of full scale, or better.  If the measurements are equal, only 8 PSK modulated power need be measured at other conditions.

Frequency Measurement

Since the maximum frequency tolerance is only 125 Hz, it is necessary to make each frequency measurement with an absolute accuracy of 1 Hz or better.  GPS or a reference traceable to NIST shall be used for calibration.

Phase Noise Measurement

A random set of symbols is sent by the test transmitter that has at least 10,000 symbols.  The phase is measured for each symbol and the average and RMS jitter determined for each of the 45-degree nominal modulation nodes.

Symbol Rate

After carrier is sent clock symbols are sent continuously.  There are two points of measurement for symbol rate on the front panel of the demodulator - the symbol strobe pulses and the "I" phase detector output.  Time between pulses should indicate 150 or 600 symbol per second rate.

Message Format

The test transmitter sends a defined message to the test receiver, That same message should be repeated at the demodulator.  This message will be sent for the three cases of the interleaver selections.

Carrier Length	 - Time between the detection of power level and first phase transition after phaselock.

Clock and MLS bits

Phase  "1" or "0" sent to the output port.  The exact transmission pattern is displayed.

Scrambling and Trellis Encoding

If the correct encoding is followed the data sent to the output port will be intelligible.  An incorrect pattern will provide a meaningless data string.  A test mode is provided  to print out the measured symbol phase in terms of phase bin.  The phase bins are numbered 0 to 71.  This is the raw data prior to any processing.

EOT Detection - This should be visible in the test mode and from the demodulator recognition mask.

Appendix B - GOES DCS Pseudo-Binary Data and Other Definitions

General

This standard specifies a standard format for Data Collection Platforms (DCPs) transmitting on random reporting channels.  The format has been structured so as to also be compatible with many self-timed (in particular) and interrogated DCPs.  The standard is based heavily on two assumptions: First, the proper interpretation and utilization of random data requires a data processing element within the data flow.  Second, that the format of all transmissions from a complying platform can be decodable through the use of a properly constructed data base which is to be contained within the data processing facility.

This standard defines the necessary attributes of both a DCP and a data base to make the data processable and useful.  The manner in which a DCP can be described by the data base determines both the format and the operating characteristics of the DCP.

DCPRS  Message Format

The DCPRS transmission format is set forth in paragraph 3.1 for the pre-amble (carrier, clock, and FSS), the GOES ID code, and Flag Word (see Figure 1).  The sensor or message data shall consist of a single 8-bit header word, followed by data from one or more sensors.  As shown in Figure 2, the header word is always a number between 0 and 63 and represents the entry number in a DCP information file which describes the format being used for that message.  Thus, a DCP is capable of transmitting up to 64 different formats and each format can be determined fully by knowing the header word and accessing a data base for that particular DCP.

| PRE-AMBLE | GOES ID 31-BIT ADDRESS | FLAG WORD | MESSAGE 1 | SP | MESSAGE 2 | SP | MESSAGE N | EOT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Figure 1.  DCPRS Transmit Format

| EIGHT BIT HEADER WORD | SENSOR 1 DATA | SENSOR 2 DATA | SENSOR N DATA |
| --- | --- | --- | --- |

Figure 2.  DCPRS Message or Sensor Data Format

The sensor data after each header word must adhere to the following requirements:

1.	Pseudo Binary Data Format

All header and sensor data will be converted to pseudo binary, regardless of its format from the sensor (analog, BCD, grey-coders, events, etc.). All data will be transmitted in a "modified ASCII" format utilizing 6- bits of an 8-bit character to represent part of each binary number.  For data requiring 12-bit precision, two consecutive modified ASCII characters are needed as shown in the example below:

| P | 1 | 211 | 210 | 29 | 28 | 27 | 26 | P | 1 | 25 | 24 | 23 | 22 | 21 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

1st DATA CHARACTER                                  2nd DATA CHARACTER

Figure 3.  12-Bit Precision Data

For 18-bit precision, three characters are required:

| P | 1 | 217 | 216 | 215 | 214 | 213 | 212 | P | 1 | 211 | 210 | 29 | 28 | 27 | 26 | P | 1 | 25 | 24 | 23 | 22 | 21 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

1st DATA CHARACTER                               2nd DATA CHARACTER                       3rd DATA CHARACTER

Figure 4.  18-Bit Precision Data

Note that bits 7 and 8 of each character are a *"one" and an odd parity bit, respectively*. Thus, data is always expressed by N characters, each character representing N x 6 bits of information.  Data within a character is transmitted least significant bit first.

*The 6-bit binary data sequence of all ones may be transmitted as 01111111 (an ASCII "DEL" character) or 10111111 (an ASCII “?” character).

2.	Signed Parameters

Many parameters, temperatures in particular, may be expressed with negative values.  In addition, the direction of change in a reading is often useful information and similar such parameter-related flags should also be handled efficiently.  Therefore, data may be expressed in one of three ways:

a)	as a positive fixed point value of precision (N x 6);

b)	as a signed value in two's complement form having a precision of (plus or minus) (N x 6 -1); or

c)	as a positive fixed point number of precision (N x 6 -1) with the high order bit used as a flag.

As an example of a negative value, a temperature value of 17 degrees below zero could be expressed with six bits as 101111.  Whereas, a signed value of *+17 degrees would be expressed as 010001.  See "Definitions" for an explanation of two's complement arithmetic.

For parameters not having negative values, but designated as being a parameter with a f lag, the high order bit is the f lag and the remaining bits are data in binary form.  The precise interpretation of the flag bit is to be defined in the DCP's associated data base.  As an example, the 11 bit precision accumulated precipitation value of 000001111011 (123) could indicate both the value of 1.23 inches (accumulated) and the fact that it is raining at the time of the measurement.  Conversely, a value of 100001111011 indicates the same reading but signifies that no perceptible change has occurred since the last sensor update.

3.	Order of Reporting

The most current data will be reported first within the DCP message.

4.	Limitations on Data Content

This standard per se, places no restriction on the number of parameters being sensed, the accuracy of the measurements, or the number of readings within a message.  The format of the message must, however, be describable by a data base (located in the receive system's computer) containing, at least, the following elements:

a)	For each parameter being reported;

1) 	Precision of the measurement being reported.  This will always be a multiple of 6 (6, 12, 18, 24, etc.) unless it is a signed parameter or has a high-order flag bit (than it is 5, 11, 17, 23, etc.).

2)	A flag indicating whether or not the data signed, or has a flag bit.

3)	Calibration coefficients which will be applied to the data (if necessary).

b)	For each possible format to be transmitted;

1)	The message format number (0-63) which corresponds to the 8-bit header word beginning each message.

2)	Parameter cycle time (in seconds or minutes) - an N/A (not appropriate) flag may be used to indicate data is not reported in cycles.

3)	Cycle offset (in minutes) -- the time delay from the end of the last complete up-date cycle, reported, to the beginning of transmission.  If this value is N/A, the data is assumed to be transmitted in real-time, or the time delay between measurement and reporting is to be reported as a parameter within the message.  This value will be *N/A for random or interrogated transmissions.

4)	A list of parameters contained within the message (or parameter cycle, if the data is reported in cycles): along with the time of the sensor update relative to the beginning of the message.  If any given parameter is updated (reported) several times within a cycle, that parameter (with the corresponding time) will be listed for each update.

If time delay is itself a reported parameter, it will be listed in the data base--the *DCP will transmit this value immediately before all parameters associated with it.

5)	Cycles per message (if appropriate).  This value indicates the number of times the listed parameters are repeated.  An N/A flag would indicate either no repeats, or an indeterminate number of parameter groups (a time delay value, with one or more data values).

6)	Multiple Messages Within A Transmission.   A transmission may contain more than one message.  Generally, multiple messages will be used when two or more formats (as defined in the data base) are needed to transmit all the desired data.  Multiple massages can also be utilized to transmit new data along with previously transmitted data --- where possible, multiple parameter cycles should be utilized in lieu of multiple messages.

Transmissions containing multiple messages will have a single ASCII space character (00100000 - LSB first) between each message.  Note: the seventh bit (underlined) is a zero and thus is not a valid data character.

7)	Bad Data.  If a sensor fails, or if for some reason the DCP is unable to transmit, proper data, an ASCII (/) character (00101111) may be substituted for each data character.  Note, the 7th bit of this character is not a one, and cannot therefore be a valid data character.

Other DCPRS Definitions

Transmission	The combination of clear radio carrier and all bits of identification, data and any special sequences sent by a DCP.

Message	Relates to all or a portion of the data segment of a transmission; the message is a segment of data that is fully defined in a DCP management data base; a transmission will contain one or more messages.

Header Word	An 8-bit character whose low order 6 bits make up a binary number that identified a format retry stored in a DCP management data base for a specific DCP.  A header word begins each message.

DCP	Contains one record per DCP that includes Data Base the Management characteristics of each parameter measured, plus a list of format entries that will identify each potential data message that the DCP can formulate and transmit.

Parameter	Data element measured by a sensor.  Common hydrometeorology parameters include stream stage, precipitation, and temperature.

Parameter	Entry of a value (composite or point) for into a  parameter DCP message.  The

Update	value may be an instantaneous value or a computed value based on many sensor values measured since the last message update.

Cycle	A procedure used by a DCP in acquiring and formatting multiple readings obtained over a period of time.  Generally a cycle consists of specific measurements taken at prescribed times within a defined interval.  A DCP may acquire and report data for several such time intervals by precisely repeating the prescribed cycle for each consecutive interval.

Two's 	A method of expressing negative numbers so that subtraction may be performed

Complement	by a simple fixed-precision binary accumulator (adder).  The negative value of a binary number is computed by complementing each bit and then adding one. (Example: The equation 4-6=-2 is computed as 4+(-6)=-2, which in 6-bit binary is 000100 + 111001 = 111101).  The magnitude of a negative value is determine by taking its two's-complement. (i.e., -(-2)=+2 or -(11101)=000010).

Appendix C - Interrogate DCP Receive Requirements

C.1	Received Frequency Characteristics  - The DCP received radio-frequency (RF) shall be as follows:

- The GOES East frequency - 468.8375 MHz

- The GOES West frequency - 468.8125 MHz

- Test/spare frequency - 468.8250 MHz

C.2.	Interrogate Signal Characteristics

- Modulation – BPSK NRZ-L

- Data Rate (user) –   1200 bps

- FEC Code –   Rate 1/2 convolutional inner code plus Reed-Solomon (255, 223) outer code.  (Same as will be used for LRIT and EMWIN on GOES N/O/P.)

- RF Data Rate –   2736 bps

- Data Filtering –   SRRC (for transmitters and receivers) with excess bandwidth factor () of 1.0  (Same as will be used for LRIT and EMWIN on GOES N/O/P.)

- Sideband Attenuation –  -30 dBc minimum

- Necessary Bandwidth –   5472 Hz

- Satellite EIRP – 40.0 dBmi

- Required bit error rate –  10-6

- Worst Case DCPRS G/T –   -22 dB/K

- Worst Case DCPRS Demod/Decode Implementation Loss –   2.0 dB

- Receiver tracking loop bandwidth –   100 Hz (minimum)

Other Parameters:

Any interested parties are requested to offer other parameters to be included in the specification, or alternatives to those suggested above.  At the very least, a definition of the digital interface, the content to be permitted, and the digital format, will need to be added to the above partial description, or any alternative that may be proposed.

APPENDIX D - GOES DCPRS TRANSMIT FREQUENCIES

| CHANNEL NUMBER | CENTER FREQUENCY | CHANNEL NUMBER | CENTER FREQUENCY | CHANNEL NUMBER | CENTER FREQUENCY |
| --- | --- | --- | --- | --- | --- |
| 1 | 401.701000 | 46 | 401.734750 | 91 | 401.768500 |
| 2 | 401.701750 | 47 | 401.735500 | 92 | 401.769250 |
| 3 | 401.702500 | 48 | 401.736250 | 93 | 401.770000 |
| 4 | 401.703250 | 49 | 401.737000 | 94 | 401.770750 |
| 5 | 401.704000 | 50 | 401.737750 | 95 | 401.771500 |
| 6 | 401.704750 | 51 | 401.738500 | 96 | 401.772250 |
| 7 | 401.705500 | 52 | 401.739250 | 97 | 401.773000 |
| 8 | 401.706250 | 53 | 401.740000 | 98 | 401.773750 |
| 9 | 401.707000 | 54 | 401.740750 | 99 | 401.774500 |
| 10 | 401.707750 | 55 | 401.741500 | 100 | 401.775250 |
| 11 | 401.708500 | 56 | 401.742250 | 101 | 401.776000 |
| 12 | 401.709250 | 57 | 401.743000 | 102 | 401.776750 |
| 13 | 401.710000 | 58 | 401.743750 | 103 | 401.777500 |
| 14 | 401.710750 | 59 | 401.744500 | 104 | 401.778250 |
| 15 | 401.711500 | 60 | 401.745250 | 105 | 401.779000 |
| 16 | 401.712250 | 61 | 401.746000 | 106 | 401.779750 |
| 17 | 401.713000 | 62 | 401.746750 | 107 | 401.780500 |
| 18 | 401.713750 | 63 | 401.747500 | 108 | 401.781250 |
| 19 | 401.714500 | 64 | 401.748250 | 109 | 401.782000 |
| 20 | 401.715250 | 65 | 401.749000 | 110 | 401.782750 |
| 21 | 401.716000 | 66 | 401.749750 | 111 | 401.783500 |
| 22 | 401.716750 | 67 | 401.750500 | 112 | 401.784250 |
| 23 | 401.717500 | 68 | 401.751250 | 113 | 401.785000 |
| 24 | 401.718250 | 69 | 401.752000 | 114 | 401.785750 |
| 25 | 401.719000 | 70 | 401.752750 | 115 | 401.786500 |
| 26 | 401.719750 | 71 | 401.753500 | 116 | 401.787250 |
| 27 | 401.720500 | 72 | 401.754250 | 117 | 401.788000 |
| 28 | 401.721250 | 73 | 401.755000 | 118 | 401.788750 |
| 29 | 401.722000 | 74 | 401.755750 | 119 | 401.789500 |
| 30 | 401.722750 | 75 | 401.756500 | 120 | 401.790250 |
| 31 | 401.723500 | 76 | 401.757250 | 121 | 401.791000 |
| 32 | 401.724250 | 77 | 401.758000 | 122 | 401.791750 |
| 33 | 401.725000 | 78 | 401.758750 | 123 | 401.792500 |
| 34 | 401.725750 | 79 | 401.759500 | 124 | 401.793250 |
| 35 | 401.726500 | 80 | 401.760250 | 125 | 401.794000 |
| 36 | 401.727250 | 81 | 401.761000 | 126 | 401.794750 |
| 37 | 401.728000 | 82 | 401.761750 | 127 | 401.795500 |
| 38 | 401.728750 | 83 | 401.762500 | 128 | 401.796250 |
| 39 | 401.729500 | 84 | 401.763250 | 129 | 401.797000 |
| 40 | 401.730250 | 85 | 401.764000 | 130 | 401.797750 |
| 41 | 401.731000 | 86 | 401.764750 | 131 | 401.798500 |
| 42 | 401.731750 | 87 | 401.765500 | 132 | 401.799250 |
| 43 | 401.732500 | 88 | 401.766250 | 133 | 401.800000 |
| 44 | 401.733250 | 89 | 401.767000 | 134 | 401.800750 |
| 45 | 401.734000 | 90 | 401.767750 | 135 | 401.801500 |

GOES DCPRS TRANSMIT FREQUENCIES (Continued)

| CHANNEL NUMBER | CENTER FREQUENCY | CHANNEL NUMBER | CENTER FREQUENCY | CHANNEL NUMBER | CENTER FREQUENCY |
| --- | --- | --- | --- | --- | --- |
| 136 | 401.802250 | 181 | 401.836000 | 226 | 401.869750 |
| 137 | 401.803000 | 182 | 401.836750 | 227 | 401.870500 |
| 138 | 401.803750 | 183 | 401.837500 | 228 | 401.871250 |
| 139 | 401.804500 | 184 | 401.838250 | 229 | 401.872000 |
| 140 | 401.805250 | 185 | 401.839000 | 230 | 401.872750 |
| 141 | 401.806000 | 186 | 401.839750 | 231 | 401.873500 |
| 142 | 401.806750 | 187 | 401.840500 | 232 | 401.874250 |
| 143 | 401.807500 | 188 | 401.841250 | 233 | 401.875000 |
| 144 | 401.808250 | 189 | 401.842000 | 234 | 401.875750 |
| 145 | 401.809000 | 190 | 401.842750 | 235 | 401.876500 |
| 146 | 401.809750 | 191 | 401.843500 | 236 | 401.877250 |
| 147 | 401.810500 | 192 | 401.844250 | 237 | 401.878000 |
| 148 | 401.811250 | 193 | 401.845000 | 238 | 401.878750 |
| 149 | 401.812000 | 194 | 401.845750 | 239 | 401.879500 |
| 150 | 401.812750 | 195 | 401.846500 | 240 | 401.880250 |
| 151 | 401.813500 | 196 | 401.847250 | 241 | 401.881000 |
| 152 | 401.814250 | 197 | 401.848000 | 242 | 401.881750 |
| 153 | 401.815000 | 198 | 401.848750 | 243 | 401.882500 |
| 154 | 401.815750 | 199 | 401.849500 | 244 | 401.883250 |
| 155 | 401.816500 | 200 | 401.850250 | 245 | 401.884000 |
| 156 | 401.817250 | 201 | 401.851000 | 246 | 401.884750 |
| 157 | 401.818000 | 202 | 401.851750 | 247 | 401.885500 |
| 158 | 401.818750 | 203 | 401.852500 | 248 | 401.886250 |
| 159 | 401.819500 | 204 | 401.853250 | 249 | 401.887000 |
| 160 | 401.820250 | 205 | 401.854000 | 250 | 401.887750 |
| 161 | 401.821000 | 206 | 401.854750 | 251 | 401.888500 |
| 162 | 401.821750 | 207 | 401.855500 | 252 | 401.889250 |
| 163 | 401.822500 | 208 | 401.856250 | 253 | 401.890000 |
| 164 | 401.823250 | 209 | 401.857000 | 254 | 401.890750 |
| 165 | 401.824000 | 210 | 401.857750 | 255 | 401.891500 |
| 166 | 401.824750 | 211 | 401.858500 | 256 | 401.892250 |
| 167 | 401.825500 | 212 | 401.859250 | 257 | 401.893000 |
| 168 | 401.826250 | 213 | 401.860000 | 258 | 401.893750 |
| 169 | 401.827000 | 214 | 401.860750 | 259 | 401.894500 |
| 170 | 401.827750 | 215 | 401.861500 | 260 | 401.895250 |
| 171 | 401.828500 | 216 | 401.862250 | 261 | 401.896000 |
| 172 | 401.829250 | 217 | 401.863000 | 262 | 401.896750 |
| 173 | 401.830000 | 218 | 401.863750 | 263 | 401.897500 |
| 174 | 401.830750 | 219 | 401.864500 | 264 | 401.898250 |
| 175 | 401.831500 | 220 | 401.865250 | 265 | 401.899000 |
| 176 | 401.832250 | 221 | 401.866000 | 266 | 401.899750 |
| 177 | 401.833000 | 222 | 401.866750 | 267 | 401.900500 |
| 178 | 401.833750 | 223 | 401.867500 | 268 | 401.901250 |
| 179 | 401.834500 | 224 | 401.868250 | 269 | 401.902000 |
| 180 | 401.835250 | 225 | 401.869000 | 270 | 401.902750 |

GOES DCPRS TRANSMIT FREQUENCIES (Continued)

| CHANNEL NUMBER | CENTER FREQUENCY | CHANNEL NUMBER | CENTER FREQUENCY | CHANNEL NUMBER | CENTER FREQUENCY |
| --- | --- | --- | --- | --- | --- |
| 271 | 401.903500 | 316 | 401.937250 | 361 | 401.971000 |
| 272 | 401.904250 | 317 | 401.938000 | 362 | 401.971750 |
| 273 | 401.905000 | 318 | 401.938750 | 363 | 401.972500 |
| 274 | 401.905750 | 319 | 401.939500 | 364 | 401.973250 |
| 275 | 401.906500 | 320 | 401.940250 | 365 | 401.974000 |
| 276 | 401.907250 | 321 | 401.941000 | 366 | 401.974750 |
| 277 | 401.908000 | 322 | 401.941750 | 367 | 401.975500 |
| 278 | 401.908750 | 323 | 401.942500 | 368 | 401.976250 |
| 279 | 401.909500 | 324 | 401.943250 | 369 | 401.977000 |
| 280 | 401.910250 | 325 | 401.944000 | 370 | 401.977750 |
| 281 | 401.911000 | 326 | 401.944750 | 371 | 401.978500 |
| 282 | 401.911750 | 327 | 401.945500 | 372 | 401.979250 |
| 283 | 401.912500 | 328 | 401.946250 | 373 | 401.980000 |
| 284 | 401.913250 | 329 | 401.947000 | 374 | 401.980750 |
| 285 | 401.914000 | 330 | 401.947750 | 375 | 401.981500 |
| 286 | 401.914750 | 331 | 401.948500 | 376 | 401.982250 |
| 287 | 401.915500 | 332 | 401.949250 | 377 | 401.983000 |
| 288 | 401.916250 | 333 | 401.950000 | 378 | 401.983750 |
| 289 | 401.917000 | 334 | 401.950750 | 379 | 401.984500 |
| 290 | 401.917750 | 335 | 401.951500 | 380 | 401.985250 |
| 291 | 401.918500 | 336 | 401.952250 | 381 | 401.986000 |
| 292 | 401.919250 | 337 | 401.953000 | 382 | 401.986750 |
| 293 | 401.920000 | 338 | 401.953750 | 383 | 401.987500 |
| 294 | 401.920750 | 339 | 401.954500 | 384 | 401.988250 |
| 295 | 401.921500 | 340 | 401.955250 | 385 | 401.989000 |
| 296 | 401.922250 | 341 | 401.956000 | 386 | 401.989750 |
| 297 | 401.923000 | 342 | 401.956750 | 387 | 401.990500 |
| 298 | 401.923750 | 343 | 401.957500 | 388 | 401.991250 |
| 299 | 401.924500 | 344 | 401.958250 | 389 | 401.992000 |
| 300 | 401.925250 | 345 | 401.959000 | 390 | 401.992750 |
| 301 | 401.926000 | 346 | 401.959750 | 391 | 401.993500 |
| 302 | 401.926750 | 347 | 401.960500 | 392 | 401.994250 |
| 303 | 401.927500 | 348 | 401.961250 | 393 | 401.995000 |
| 304 | 401.928250 | 349 | 401.962000 | 394 | 401.995750 |
| 305 | 401.929000 | 350 | 401.962750 | 395 | 401.996500 |
| 306 | 401.929750 | 351 | 401.963500 | 396 | 401.997250 |
| 307 | 401.930500 | 352 | 401.964250 | 397 | 401.998000 |
| 308 | 401.931250 | 353 | 401.965000 | 398 | 401.998750 |
| 309 | 401.932000 | 354 | 401.965750 | 399 | 401.999500 |
| 310 | 401.932750 | 355 | 401.966500 | 400 | 402.000250 |
| 311 | 401.933500 | 356 | 401.967250 | 401 | 402.001000 |
| 312 | 401.934250 | 357 | 401.968000 | 402 | 402.001750 |
| 313 | 401.935000 | 358 | 401.968750 | 403 | 402.002500 |
| 314 | 401.935750 | 359 | 401.969500 | 404 | 402.003250 |
| 315 | 401.936500 | 360 | 401.970250 | 405 | 402.004000 |

GOES DCPRS TRANSMIT FREQUENCIES (Continued)

| CHANNEL NUMBER | CENTER FREQUENCY | CHANNEL NUMBER | CENTER FREQUENCY | CHANNEL NUMBER | CENTER FREQUENCY |
| --- | --- | --- | --- | --- | --- |
| 406 | 402.004750 | 451 | 402.038500 | 496 | 402.072250 |
| 407 | 402.005500 | 452 | 402.039250 | 497 | 402.073000 |
| 408 | 402.006250 | 453 | 402.040000 | 498 | 402.073750 |
| 409 | 402.007000 | 454 | 402.040750 | 499 | 402.074500 |
| 410 | 402.007750 | 455 | 402.041500 | 500 | 402.075250 |
| 411 | 402.008500 | 456 | 402.042250 | 501 | 402.076000 |
| 412 | 402.009250 | 457 | 402.043000 | 502 | 402.076750 |
| 413 | 402.010000 | 458 | 402.043750 | 503 | 402.077500 |
| 414 | 402.010750 | 459 | 402.044500 | 504 | 402.078250 |
| 415 | 402.011500 | 460 | 402.045250 | 505 | 402.079000 |
| 416 | 402.012250 | 461 | 402.046000 | 506 | 402.079750 |
| 417 | 402.013000 | 462 | 402.046750 | 507 | 402.080500 |
| 418 | 402.013750 | 463 | 402.047500 | 508 | 402.081250 |
| 419 | 402.014500 | 464 | 402.048250 | 509 | 402.082000 |
| 420 | 402.015250 | 465 | 402.049000 | 510 | 402.082750 |
| 421 | 402.016000 | 466 | 402.049750 | 511 | 402.083500 |
| 422 | 402.016750 | 467 | 402.050500 | 512 | 402.084250 |
| 423 | 402.017500 | 468 | 402.051250 | 513 | 402.085000 |
| 424 | 402.018250 | 469 | 402.052000 | 514 | 402.085750 |
| 425 | 402.019000 | 470 | 402.052750 | 515 | 402.086500 |
| 426 | 402.019750 | 471 | 402.053500 | 516 | 402.087250 |
| 427 | 402.020500 | 472 | 402.054250 | 517 | 402.088000 |
| 428 | 402.021250 | 473 | 402.055000 | 518 | 402.088750 |
| 429 | 402.022000 | 474 | 402.055750 | 519 | 402.089500 |
| 430 | 402.022750 | 475 | 402.056500 | 520 | 402.090250 |
| 431 | 402.023500 | 476 | 402.057250 | 521 | 402.091000 |
| 432 | 402.024250 | 477 | 402.058000 | 522 | 402.091750 |
| 433 | 402.025000 | 478 | 402.058750 | 523 | 402.092500 |
| 434 | 402.025750 | 479 | 402.059500 | 524 | 402.093250 |
| 435 | 402.026500 | 480 | 402.060250 | 525 | 402.094000 |
| 436 | 402.027250 | 481 | 402.061000 | 526 | 402.094750 |
| 437 | 402.028000 | 482 | 402.061750 | 527 | 402.095500 |
| 438 | 402.028750 | 483 | 402.062500 | 528 | 402.096250 |
| 439 | 402.029500 | 484 | 402.063250 | 529 | 402.097000 |
| 440 | 402.030250 | 485 | 402.064000 | 530 | 402.097750 |
| 441 | 402.031000 | 486 | 402.064750 | 531 | 402.098500 |
| 442 | 402.031750 | 487 | 402.065500 | 532 | 402.099250 |
| 443 | 402.032500 | 488 | 402.066250 |  |  |
| 444 | 402.033250 | 489 | 402.067000 |  |  |
| 445 | 402.034000 | 490 | 402.067750 |  |  |
| 446 | 402.034750 | 491 | 402.068500 |  |  |
| 447 | 402.035500 | 492 | 402.069250 |  |  |
| 448 | 402.036250 | 493 | 402.070000 |  |  |
| 449 | 402.037000 | 494 | 402.070750 |  |  |
| 450 | 402.037750 | 495 | 402.071500 |  |  |

Appendix E - GOES DCS Certification Test Notes

Link Phase Noise Budget

A phase noise budget is proposed for the entire DCS so that each part of the system can be individually tested, independent of the other parts, and still be assured that all components that meet their assigned requirement will provide acceptable system function.

The proposed budget for the DCPRS is derived as follows:

- The demodulators are expected to normally operate with a bit error rate near 1 in 107, and it is desired that errors due to phase noise should be about two orders of magnitude better, i.e., about 1 in 109.

- Phase noise is a normal or Gaussian distributed variable, with standard deviation, .  Therefore the dual-tail, Normal Error Integral gives a probability of 1 in 109 that it exceeds 6.1 from its mean value.

- The total phase error required to cause bit errors in an 8PSK demodulator is 22.5 degrees, half of the 45 degree phase shift between each of the nominal phase states.

- This document proposes a DCPRS phase offset/bias 1.0 degrees.

- 22.5 - 1.0 = 21.5 and 21.5 / 6.1 = 3.52

- Therefore, the total system phase noise must be 3.5 degrees or less.

- The phase noise in the GOES N primary and redundant DCPR transponders has been measured at  0.5 degrees, worst case.  Therefore an allocation of 1.0 degrees is proposed for all of the GOES N and GOES R series of satellites.

- The DCPRS phase noise required by the current Certification Standards is 2.5 degrees.

- The phase noise allocation proposed for the dual down conversion receive path at WCDAS is 2.0 degrees.

- The phase noise allocation proposed for the additional down conversion at the input to the demodulators at WCDAS is 1.0 degrees.

- The RSS for all these components of the system phase noise is 3.5 degrees.

The integration limits for calculating phase noise are 10 Hz and 150 Hz for the 300 bps channels and 36 Hz and 600 Hz for the 1200 bps channels.  These are the values that will apply to the demodulators at WCDAS and have been set as the standard to be used for the satellite and DCPRS requirements.  It is recommended that these values also be used in the design of future demodulators for WCDAS and/or DRGS, at these data rates.