DERIVATION OF THE DESIGN MAXIMUM LOADING FOR DCPR

Prepared by:

Peter Woolner

Mitretek Systems

March 6, 2006

Summary of the Design Maximum Loading

The worst case design channel loading for DCPR transponder is defined as 205 channels at an average uplink EIRP of 48.5 dBmi.  This is a theoretical estimate of the average power levels for the modulation types and data rates transmitted by the platforms and is deemed by NOAA to represent the worst case practical load set for this transponder.

Basic Channel Assignment Rules:

- The spectrum from 401.7 MHz to 402.1 MHz is divided into 532 designated channels, 750 Hz wide, with center frequencies from 401.70100 MHz to 402.099250 MHz, designated 1 through 532.

- Each 300 bps user will occupy one of these channels.

- Each 1200 bps user will occupy three of these channels.

- Odd channels are assigned to GOES-East and even channels to GOES-West.

- Assignments for 300 bps users shall be channels 1 through 369.

- Channels 371, 374, 377, 380, 383, 386, 389, 392, 395, and 398 are for 1200 bps.

- Channel 401 (equivalent to old channel 201 at 402.00100 MHz) and above are in the band used by transients and it is assumed these users will remain at 100 bps.

- It is also assumed that 10 CDMA channels can be accommodated per satellite.  (It is not yet sure that CDMA can share the DCPR transponder, but 10 channels per satellite is assumed for purposes of calculating the worst case loading.)

Proposed Derivation of Worst Case Channel Loading:

- Each satellite is allocated 185 channels at 300 bps, 5 channels at 1200 bps, and 10 CDMA channels.  (GOES west actually has only 184 channels at 300 bps.)

- For the worst case instant, only 80% of these channels will be active.

- In addition, the worst case channel loading shall include 4 channels on each satellite at 100 bps for transient users.

- Channels allocated to one satellite are assumed to also illuminate the other satellite at a level, on average, 6 dB below the level into the allocated satellite.

- The design range of uplink EIRP values is:

  - For 1200 bps transmissions, 50 to 52 dBmi

  - For 300 bps transmissions, 47 to 49 dBmi

  - For 100 bps transmissions, 40 to 50 dBmi

  - For CDMA transmissions, 44 to 46 dBmi

The distribution assumed for worst case loading is shown in the table below.

- Transmission at EIRP values greater than listed above is not permitted.

- Transmission at EIRP values less than listed above is allowed, but users should be aware that variations between satellites and of the instantaneous transponder loading may cause unacceptable error rates to users with low EIRP.

WORST CASE LOADING FOR THE DCPR TRANSPONDER

| Data Rate bps | Uplink Transmit EIRP |  | Active Chan per Satellite | Total EIRP Watts |
| --- | --- | --- | --- | --- |
|  | dBm | Watts |  |  |
| 1200 | 52 | 158.49 | 2 | 316.98 |
| 1200 | 51 | 125.89 | 1 | 125.89 |
| 1200 | 50 | 100.00 | 1 | 100.00 |
| 300 | 49 | 79.43 | 65 | 5162.95 |
| 300 | 48.5 | 70.79 | 45 | 3185.55 |
| 300 | 48 | 63.10 | 25 | 1577.50 |
| 300 | 47 | 50.12 | 13 | 651.56 |
| 100 | 50 | 100.00 | 1 | 100.00 |
| 100 | 47 | 50.12 | 1 | 50.12 |
| 100 | 44 | 25.12 | 1 | 25.12 |
| 100 | 41 | 12.59 | 1 | 12.59 |
| CDMA | 46 | 39.81 | 4 | 159.24 |
| CDMA | 45 | 31.62 | 2 | 63.24 |
| CDMA | 44 | 25.12 | 2 | 50.24 |
|  |  | TOTALS | 164 | 11580.98 |
|  | Average EIRP per channel (Watts) : |  |  | 70.62 |
|  | Average EIRP per channel (dBmi) : |  |  | 48.5 |
|  |  |  | No of Active Channels | Average EIRP (dBmi) |
| EQUIVALENT SPILLOVER LOADING FROM THE OTHER SATELLITE (= ¼ DESIRED SATELLITE) |  |  | 41 | 48.5 |
| TOTAL LOADING PER SATELLITE : |  |  | 205 | 48.5 |

NESDIS expects the performance of links within the above uplink EIRP limits, and using a ground receive system G/T of 15 dB/K or more, will provide error rates better than one in 106 bits, assuming normal propagation conditions and equipment that meets the NESDIS Certification Standards.

SPECIAL NOTE

NESDIS may choose to revise this estimate of the design maximum loading at any time if actual measurements of performance or usage plans are significantly changed.